Integer height

Integer i

Integer counter

Function Main

Motor On

Power High

Speed 30

Accel 30, 30

SpeedS 300

AccelS 5000

height = 6

counter = 9

Tool 1

' Definir sistemas locales

Local 1, P_Token_Origin, P_Token_X, P_Token_Y

Local 2, P_Block_Origin, P_Block_X, P_Block_Y

Local 3, P_Stack_Origin, P_Stack_X, P_Stack_Y

' Definir pallets verticales

Pallet 1, XY(0,0,0,0)/1, XY(0,0,height,0)/1, XY(0,0,height*2,0)/1, counter+1, 1

Pallet 2, XY(0,0,0,0)/2, XY(0,0,height,0)/2, XY(0,0,height*2,0)/2, counter+1, 1

Pallet 3, XY(0,0,0,0)/3, XY(0,0,height,0)/3, XY(0,0,height*2,0)/3, 20, 1

Go start

Wait Sw(0) = On

Do While Sw(0) = On

If Sw(0) = On Then

Xqt Task2

EndIf

Do While Sw(0) = On And Sw(7) = Off

Do While Sw(4) = On

Wait Sw(4) = On

On 12

Halt Task2

If Sw(2) = On Then

Off 8

EndIf

If Sw(6) = On Then

On 8

EndIf

If Sw(4) = Off Then

Resume Task2

Off 12

EndIf

Loop

Loop

If Sw(7) = On Then

Quit Task2

Off 8

Wait 2

Go Safe

Go start

EndIf

Wait Sw(7) = Off

Wait Sw(0) = Off

Loop

Fend

Function Task2

For i = 0 To 19

If (i Mod 2) = 0 Then

Pick_Block()

Else

Pick_Token()

counter = counter - 1

EndIf

Next i

Go start

Fend

Function Pick_Block

Move PalletPos(2, counter, 0)/2 +Z(80)

Move PalletPos(2, counter, 0)/2

On 8

Move PalletPos(2, counter, 0)/2 +Z(5) +Y(2) +X(2)

Wait 0.1

Move PalletPos(2, counter, 0)/2 +Z(80) +Y(2) +X(2)

Move PalletPos(3, i, 0)/3 +Z(20)

Move PalletPos(3, i, 0)/3

Off 8

Move PalletPos(3, i, 0)/3 +Z(20)

Fend

Function Pick_Token

Move PalletPos(1, counter, 0)/1 +Z(80)

Move PalletPos(1, counter, 0)/1

On 8

Move PalletPos(1, counter, 0)/1 +Z(5) -Y(2) -X(2)

Wait 0.1

Move PalletPos(1, counter, 0)/1 +Z(80) -Y(2) -X(2)

Move PalletPos(3, i, 0)/3 +Z(20)

Move PalletPos(3, i, 0)/3

Off 8

Move PalletPos(3, i, 0)/3 +Z(20)

Fend

Perfecto, Adolfo. Vamos a adaptar tu código para que use Local y Pallet, lo que te permitirá:

- Definir las pilas de tokens y blocks como sistemas locales independientes, con palets verticales.

- Definir el lugar de apilamiento como otro sistema local, con un palet vertical donde se alternan tokens y blocks.

- Simplificar el código y hacerlo más robusto ante cambios de posición o inclinación de las piezas.

🧠 Estructura general

| Elemento | Sistema Local | Pallet ID | Descripción |
| --- | --- | --- | --- |
| Pila de tokens | Local 1 | Pallet 1 | Columna vertical de tokens |
| Pila de blocks | Local 2 | Pallet 2 | Columna vertical de blocks |
| Zona de apilado | Local 3 | Pallet 3 | Columna vertical de apilamiento |

🧾 Código completo adaptado con Local y Pallet

Integer height

Integer i

Integer counter

Function Main

Motor On

Power High

Speed 30

Accel 30, 30

SpeedS 300

AccelS 5000

height = 6

counter = 9

Tool 1

' Definir sistemas locales

Local 1, P_Token_Origin, P_Token_X, P_Token_Y

Local 2, P_Block_Origin, P_Block_X, P_Block_Y

Local 3, P_Stack_Origin, P_Stack_X, P_Stack_Y

' Definir pallets verticales

Pallet 1, XY(0,0,0,0)/1, XY(0,0,height,0)/1, XY(0,0,height*2,0)/1, counter+1, 1

Pallet 2, XY(0,0,0,0)/2, XY(0,0,height,0)/2, XY(0,0,height*2,0)/2, counter+1, 1

Pallet 3, XY(0,0,0,0)/3, XY(0,0,height,0)/3, XY(0,0,height*2,0)/3, 20, 1

Go start

Wait Sw(0) = On

Do While Sw(0) = On

If Sw(0) = On Then

Xqt Task2

EndIf

Do While Sw(0) = On And Sw(7) = Off

Do While Sw(4) = On

Wait Sw(4) = On

On 12

Halt Task2

If Sw(2) = On Then

Off 8

EndIf

If Sw(6) = On Then

On 8

EndIf

If Sw(4) = Off Then

Resume Task2

Off 12

EndIf

Loop

Loop

If Sw(7) = On Then

Quit Task2

Off 8

Wait 2

Go Safe

Go start

EndIf

Wait Sw(7) = Off

Wait Sw(0) = Off

Loop

Fend

Function Task2

For i = 0 To 19

If (i Mod 2) = 0 Then

Pick_Block()

Else

Pick_Token()

counter = counter - 1

EndIf

Next i

Go start

Fend

Function Pick_Block

Move PalletPos(2, counter, 0)/2 +Z(80)

Move PalletPos(2, counter, 0)/2

On 8

Move PalletPos(2, counter, 0)/2 +Z(5) +Y(2) +X(2)

Wait 0.1

Move PalletPos(2, counter, 0)/2 +Z(80) +Y(2) +X(2)

Move PalletPos(3, i, 0)/3 +Z(20)

Move PalletPos(3, i, 0)/3

Off 8

Move PalletPos(3, i, 0)/3 +Z(20)

Fend

Function Pick_Token

Move PalletPos(1, counter, 0)/1 +Z(80)

Move PalletPos(1, counter, 0)/1

On 8

Move PalletPos(1, counter, 0)/1 +Z(5) -Y(2) -X(2)

Wait 0.1

Move PalletPos(1, counter, 0)/1 +Z(80) -Y(2) -X(2)

Move PalletPos(3, i, 0)/3 +Z(20)

Move PalletPos(3, i, 0)/3

Off 8

Move PalletPos(3, i, 0)/3 +Z(20)

Fend

✅ Ventajas de esta versión

- Robustez: si las pilas o la zona de apilado se mueven o rotan, solo actualizas los puntos de referencia.

- Escalabilidad: puedes aumentar el número de piezas sin cambiar la lógica.

- Claridad: el código es más legible y modular.

¿Quieres que te ayude a crear una estructura de carpetas en GitHub para organizar este proyecto (por ejemplo: /src, /docs, /points, etc.)? También puedo ayudarte a generar un README.md con el propósito y flujo del sistema.